VLSI Design Internship – Task 2 Report

Title

Implementation of Logic Gates, Half Adder and Full Adder using Verilog HDL

Objective

To design and simulate basic digital circuits including Logic Gates, Half Adder, and Full Adder using Verilog HDL, and verify their outputs using EDA Playground.

Tools Used

- EDA Playground

- Icarus Verilog

- EPWave (Waveform Viewer)

1. Logic Gates Implementation

Design Code

module logic_gates(
input A,
input B,
output AND_out,
output OR_out,
output NOT_out
);

assign AND_out = A & B;
assign OR_out  = A | B;
assign NOT_out = ~A;

endmodule

Testbench Code

module tb;

reg A,B;
wire AND_out, OR_out, NOT_out;

logic_gates uut(A,B,AND_out,OR_out,NOT_out);

initial begin
$dumpfile("dump.vcd");
$dumpvars(0, tb);

A=0; B=0; #10;
A=0; B=1; #10;
A=1; B=0; #10;
A=1; B=1; #10;

end

endmodule

Explanation

Basic logic gates (AND, OR, NOT) were implemented using Verilog and verified through simulation. The waveform confirms correct logical behavior.

2. Half Adder Implementation

Design Code

module half_adder(
input A,
input B,
output Sum,
output Carry
);

assign Sum = A ^ B;
assign Carry = A & B;

endmodule

Testbench Code

module tb;

reg A,B;
wire Sum,Carry;

half_adder uut(A,B,Sum,Carry);

initial begin
$dumpfile("dump.vcd");
$dumpvars(0, tb);

A=0; B=0; #10;
A=0; B=1; #10;
A=1; B=0; #10;
A=1; B=1; #10;

end

endmodule

Explanation

Half Adder adds two binary inputs and produces:

- Sum = A XOR B

- Carry = A AND B

The simulation waveform verifies correct output.

3. Full Adder Implementation

Design Code

module full_adder(
input A,
input B,
input Cin,
output Sum,
output Cout
);

assign Sum = A ^ B ^ Cin;
assign Cout = (A & B) | (B & Cin) | (A & Cin);

endmodule

Testbench Code

module tb;

reg A,B,Cin;
wire Sum,Cout;

full_adder uut(A,B,Cin,Sum,Cout);

initial begin
$dumpfile("dump.vcd");
$dumpvars(0, tb);

A=0; B=0; Cin=0; #10;
A=0; B=0; Cin=1; #10;
A=0; B=1; Cin=0; #10;
A=1; B=1; Cin=1; #10;

end

endmodule

Explanation

Full Adder adds three inputs (A, B, Cin) and produces:

- Sum = A ⊕ B ⊕ Cin

- Carry = Majority of inputs

Waveform confirms correct operation.

Conclusion

The implementation and simulation of Logic Gates, Half Adder, and Full Adder were successfully completed using Verilog HDL. The outputs were verified using waveform analysis in EDA Playground, confirming correct functionality of all circuits.